# EV Battery Intelligence Platform

Project & Research Documentation

Research-backed architecture, AI/ML roadmap, datasets, and literature review
Version 1.0 | August 2026

# 1. Executive Summary

The EV Battery Intelligence Platform is an intelligent, research-oriented battery analytics and decision-support system designed to extend a conventional Battery Management System (BMS). Instead of only displaying voltage, current, temperature and State of Charge (SOC), the platform combines telemetry, physics-based estimation, machine learning, anomaly detection, battery degradation modelling, Remaining Useful Life (RUL) prediction, thermal intelligence and explainable analytics into a unified digital platform.

The core research direction is a hybrid physics + data-driven architecture. Conventional battery models and filters provide physically constrained estimates, while ML/DL models learn nonlinear patterns that are difficult to capture with a fixed equivalent-circuit model. This is especially important because recent literature continues to identify generalization, data scarcity, interpretability, computational cost and safety validation as major challenges.

# 2. Problem Statement

Traditional BMS implementations are excellent at real-time protection and monitoring, but many advanced functions remain difficult to implement reliably: accurate SOC under dynamic loads, SOH estimation before measurable capacity loss becomes obvious, RUL prediction, early fault detection, thermal-risk forecasting and actionable maintenance recommendations.

The project therefore asks: Can heterogeneous battery telemetry be transformed into a continuously updated digital representation of battery condition, future degradation and risk, while preserving physical consistency and providing explanations that an engineer can understand?

# 3. Project Objectives

Ingest and validate cell/pack telemetry such as voltage, current, temperature, SOC, cycle count and operating conditions.

Estimate SOC using a robust state-estimation layer combining battery models, Kalman filtering and/or ML.

Estimate SOH from capacity/resistance/degradation features and long-term operating history.

Predict RUL and degradation trajectory with uncertainty rather than a single unexplained number.

Detect abnormal operating conditions, sensor anomalies and battery fault signatures.

Model thermal behaviour and identify overheating / thermal-risk patterns.

Create a battery digital twin / virtual representation that is updated from telemetry.

Expose prediction and risk results through APIs and an engineering dashboard.

Provide explainable insights: why a risk score changed, which signals contributed, and what action is recommended.

Build a reproducible research pipeline with benchmark datasets and measurable evaluation metrics.

# 4. Proposed System Architecture

| Layer | Responsibilities |
| --- | --- |
| Data acquisition | CAN/OBD/telemetry simulator, CSV/Parquet streams, BMS logs, public battery datasets. |
| Data quality & preprocessing | Timestamp alignment, missing-value handling, outlier detection, sensor validation, normalization and cycle segmentation. |
| Physics/state estimation | Equivalent-circuit model, Coulomb counting, OCV-SOC mapping, EKF/UKF and thermal/state observers. |
| Feature & signal layer | dV/dt, dT/dt, internal-resistance proxies, charge/discharge statistics, cycle features, temperature statistics and temporal windows. |
| AI/ML layer | SOC/SOH models, RUL forecasting, anomaly detection, fault classification and thermal prediction. |
| Risk & intelligence layer | Risk aggregation, confidence/uncertainty, trend analysis, alert prioritization and maintenance recommendations. |
| Digital twin | Battery pack/cell state, degradation trajectory, estimated internal states and what-if simulations. |
| API & application layer | REST/WebSocket APIs, dashboards, alerts, historical analytics and downstream interfaces. |

# 5. AI/ML Modules

| Module | Recommended approach | Evaluation |
| --- | --- | --- |
| SOC estimation | Baseline: Coulomb counting + OCV/ECM + EKF/UKF. Advanced: LSTM/GRU/TCN or hybrid PINN/state observer. | MAE/RMSE, robustness across temperature and drive cycles. |
| SOH estimation | Capacity/resistance proxies + gradient boosting, temporal DL or physics-informed model. | MAE/RMSE, cross-cell generalization, monotonicity. |
| RUL prediction | Sequence models, degradation-trajectory models, survival/prognostic models and uncertainty estimation. | RMSE/MAE, prediction horizon, uncertainty calibration. |
| Anomaly detection | Isolation Forest/One-Class methods, autoencoders, residual-based detection and physics residuals. | Precision, recall, F1, false alarm rate, detection lead time. |
| Thermal prediction | Regression/sequence model or reduced-order thermal model; optional predictive control. | Temperature MAE/RMSE and maximum error. |
| Risk scoring | Rule + statistical + ML evidence aggregation with confidence weighting. | Calibration, false positives, lead time and engineer interpretability. |

# 6. Research Direction: Hybrid Physics + AI

The strongest research direction for this project is not to replace battery physics with a black-box neural network. Instead, use physics to constrain learning and use ML to model residual/nonlinear behaviour. Examples include:

ECM + EKF for a stable baseline SOC estimator, followed by an ML residual model.

Physics-informed loss terms that penalize physically impossible SOC/SOH trajectories.

Monotonic degradation constraints so SOH does not increase without a justified recovery mechanism.

Physics residuals as anomaly signals: disagreement between measured behaviour and expected model behaviour.

Thermal and electrical state estimates fused into a common battery state vector.

Uncertainty-aware predictions so the dashboard can distinguish high confidence from extrapolation.

# 7. Suggested Database Model

battery_pack: pack_id, chemistry, nominal_capacity, nominal_voltage, configuration, metadata

cell: cell_id, pack_id, position, nominal_capacity, manufacturing metadata

telemetry: timestamp, pack_id/cell_id, voltage, current, temperature, SOC, charger/load state

cycle: cycle_id, battery_id, start/end timestamps, charge/discharge statistics

state_estimate: timestamp, SOC, SOH, SOE, SOP, estimated internal resistance, confidence

prediction: model_id, timestamp, RUL, degradation trajectory, prediction interval, confidence

anomaly_event: event_id, timestamp, severity, type, evidence, confidence, status

model_registry: model version, dataset version, training date, metrics, feature schema

alerts: severity, trigger, recommended action, acknowledgement state

# 8. Dataset Strategy

A practical research workflow should start with public datasets and then move toward simulated or real BMS telemetry. NASA's Li-ion Battery Aging Dataset contains charge, discharge and EIS measurements at different temperatures and loads, with experiments continued until a defined capacity-fade end-of-life criterion. It is useful for SOH/RUL benchmarking.

| Dataset | Use | Priority |
| --- | --- | --- |
| NASA Li-ion Battery Aging Dataset | SOH, RUL, degradation trends, feature engineering | High priority |
| CALCE battery datasets | Cell ageing, cycle-life and degradation benchmarking | High priority |
| Oxford Battery Degradation Dataset | Degradation modelling and cycle-life research | High priority |
| MATR / public EV battery datasets | Real-world or dynamic operating-condition research where licensing permits | Useful |
| Project-generated synthetic telemetry | Integration testing, anomaly scenarios and digital-twin demonstrations | Essential for MVP |

# 9. Research Papers Most Relevant to the Project

| Paper | Why it matters | DOI / source |
| --- | --- | --- |
| Deep learning enabled state of charge, state of health and remaining useful life estimation for smart battery management system: Methods, implementations, issues and prospects | Journal of Energy Storage, 2022. Review of DL approaches for SOC, SOH and RUL; useful as the overall ML/BMS literature foundation. | https://doi.org/10.1016/j.est.2022.105752 |
| Predicting the state of charge and health of batteries using data-driven machine learning | Nature Machine Intelligence, 2020. Strong foundational paper for data-driven SOC/SOH modelling. | https://doi.org/10.1038/s42256-020-0156-7 |
| Recent developments and challenges in state-of-charge estimation techniques for electric vehicle batteries: A review | Journal of Energy Storage, 2024. Compares conventional, filter-based, data-driven and hybrid SOC methods. | https://doi.org/10.1016/j.est.2024.113623 |
| Electric vehicle battery capacity degradation and health estimation using machine-learning techniques: a review | Clean Energy, 2023. Focuses on SOH/RUL and ML approaches for EV battery ageing. | https://doi.org/10.1093/ce/zkad047 |
| A comprehensive review of machine learning-based state of health estimation for lithium-ion batteries: data, features, algorithms, and future challenges | Renewable and Sustainable Energy Reviews, 2025. Particularly useful for feature engineering, model selection and generalization issues. | https://doi.org/10.1016/j.rser.2025.116125 |
| State estimation for electric vehicles with deep learning- a survey | Journal of Power Sources, 2026. Broad survey covering SOC, SOH, SOE, SOP and temperature/state estimation. | https://www.sciencedirect.com/science/article/pii/S0378779626000933 |
| Machine learning is rewiring Li-ion battery RUL prediction: A systematic review and outlook | Journal of Energy Chemistry, 2026. Useful for RUL taxonomy, evaluation and current research gaps. | https://doi.org/10.1016/j.jechem.2026.05.023 |
| Physics-informed neural networks for battery modeling and state estimation: A methodological review | Renewable and Sustainable Energy Reviews, 2026. Excellent foundation for the project's physics-informed AI direction. | https://doi.org/10.1016/j.rser.2026.117226 |
| State-of-charge estimation of batteries using parameterized physics-informed neural networks | Journal of Energy Storage, 2026. Directly relevant to robust SOC estimation under diverse conditions and limited data. | https://doi.org/10.1016/j.est.2026.122235 |
| Physics-informed neural networks for electrode-level state estimation in lithium-ion batteries | Journal of Power Sources, 2021. Demonstrates physics-informed learning for internal battery state estimation. | https://doi.org/10.1016/j.jpowsour.2021.230034 |
| Hybrid Modeling of Lithium-Ion Battery: Physics-Informed Neural Network for Battery State Estimation | Batteries, 2023. Useful example of combining physical battery modelling with PINNs. | https://doi.org/10.3390/batteries9060301 |
| A time series physics-informed neural network framework for the state of health estimation of lithium-ion batteries | Journal of Energy Storage, 2026. Directly relevant to time-series SOH modelling and physics-informed degradation learning. | https://doi.org/10.1016/j.est.2025.119549 |
| SOH Estimation of Lithium-Ion Batteries Using a Residual Multilayer Perceptron-Based, Physics-Informed Neural Network for the Battery Management System | Batteries, 2026. Relevant to monotonic SOH constraints and NASA dataset benchmarking. | https://www.mdpi.com/2313-0105/12/8/294 |
| Machine learning-based predictive control of thermal management system in battery electric vehicles | 2025/2026 research on ML predictive thermal management; useful for the thermal intelligence module. | https://www.sciencedirect.com/science/article/pii/S2451904925008959 |
| Deep reinforcement learning-based thermal management of battery subpack in electric vehicle | 2026. Relevant if the project later evolves from prediction into active thermal control. | https://doi.org/10.1016/j.compchemeng.2025.109406 |

# 10. Research Paper → Project Module Mapping

| Project area | Recommended literature | Implementation path |
| --- | --- | --- |
| SOC estimation | 2024 SOC review; 2026 parameterized PINN; 2026 deep-learning state-estimation survey | Baseline EKF → LSTM/GRU → physics-informed hybrid |
| SOH estimation | 2023 EV degradation review; 2025 SOH ML review; 2026 TS-PINN | Capacity/resistance features → temporal ML → PINN |
| RUL prediction | 2022 smart BMS review; 2026 RUL systematic review | Sequence forecasting + uncertainty + degradation trajectory |
| Physics-informed AI | 2021 electrode PINN; 2023 hybrid PINN; 2026 PINN methodological review | Physics loss, residual learning, constrained states |
| Thermal intelligence | ML predictive thermal management; 2026 DRL thermal management | Thermal prediction first; control only as later research |
| Fault/anomaly detection | Smart-BMS DL review + physics-informed state estimation literature | Model residuals + autoencoder/Isolation Forest + supervised fault model |

# 11. Research Gaps the Project Can Target

Cross-temperature and cross-cell generalization: many models perform well only on conditions represented during training.

Data scarcity: real EV battery datasets are much harder to obtain than laboratory cycle data.

Physics consistency: purely data-driven models can produce implausible state trajectories.

Uncertainty: a prediction without confidence/interval information is difficult to use for engineering decisions.

Early warning: useful systems should detect degradation/fault trends before hard threshold violations.

Explainability: the platform should expose signal contributions and physical evidence rather than only a risk score.

Deployment constraints: models must eventually be optimized for latency, memory and edge/vehicle execution.

Evaluation leakage: battery-cycle datasets require careful train/test splitting by battery/cell to avoid overly optimistic results.

# 12. Recommended Experimental Plan

1. Build a deterministic baseline using Coulomb counting + OCV/ECM + EKF.

2. Create a reproducible preprocessing and cycle-segmentation pipeline.

3. Benchmark classical ML models for SOH and anomaly detection.

4. Train an LSTM/GRU or TCN baseline for SOC/SOH/RUL.

5. Add physics constraints and compare against the unconstrained model.

6. Perform cross-temperature and cross-battery validation.

7. Add uncertainty estimation and calibration.

8. Integrate the inference services into the backend and dashboard.

9. Run end-to-end tests from telemetry ingestion to alert generation.

10. Document model cards, dataset versions, metrics and known failure modes.

# 13. Evaluation Metrics

SOC: MAE, RMSE, maximum absolute error, dynamic-cycle error, temperature-wise error.

SOH: MAE, RMSE, R², monotonicity violations, cross-cell generalization.

RUL: MAE/RMSE, prediction horizon, confidence interval coverage and calibration.

Anomaly detection: precision, recall, F1, false alarm rate, detection lead time.

Thermal: MAE/RMSE, maximum temperature error and threshold-crossing prediction error.

System: API latency, inference latency, throughput, data-loss rate and alert latency.

# 14. Suggested Technology Stack

| Area | Recommended technologies |
| --- | --- |
| Frontend | React + TypeScript + modern charting/visualization |
| Backend | Python + FastAPI |
| ML | PyTorch / scikit-learn; optional XGBoost; MLflow for experiment/model tracking |
| Physics | NumPy/SciPy + custom ECM/EKF/UKF modules; optional PyBaMM for electrochemical modelling |
| Data | PostgreSQL + TimescaleDB or equivalent time-series storage; object storage for datasets |
| Streaming | WebSocket/MQTT/Kafka depending on scale |
| Deployment | Docker; CI/CD; optional GPU inference; edge deployment later |
| Observability | Structured logging, metrics, model/data drift monitoring |

# 15. MVP vs Research Version

| Stage | Scope |
| --- | --- |
| MVP | Telemetry ingestion, dashboard, baseline SOC, basic SOH, anomaly detection, historical charts, REST/WebSocket APIs. |
| Research V1 | LSTM/GRU SOC/SOH, RUL, cross-condition evaluation, model registry, uncertainty and explainability. |
| Research V2 | Hybrid physics + AI, PINN/physics residuals, digital twin, thermal prediction and advanced fault diagnosis. |
| Advanced | Online learning, fleet-level analytics, federated learning, predictive thermal control and edge deployment. |

# 16. Key Takeaway

The project is strongest when positioned as an intelligent battery analytics and prognostics platform rather than simply an ML model. The research contribution can come from a reproducible hybrid architecture that combines physical state estimation, temporal ML, physics-informed constraints, uncertainty, anomaly detection and an explainable risk layer. The literature reviewed above provides a clear path from conventional BMS estimation to modern physics-informed battery intelligence.

# 17. Core References / Starting Reading List

Deep learning enabled state of charge, state of health and remaining useful life estimation for smart battery management system: Methods, implementations, issues and prospects. Journal of Energy Storage, 2022. Review of DL approaches for SOC, SOH and RUL; useful as the overall ML/BMS literature foundation. https://doi.org/10.1016/j.est.2022.105752

Predicting the state of charge and health of batteries using data-driven machine learning. Nature Machine Intelligence, 2020. Strong foundational paper for data-driven SOC/SOH modelling. https://doi.org/10.1038/s42256-020-0156-7

Recent developments and challenges in state-of-charge estimation techniques for electric vehicle batteries: A review. Journal of Energy Storage, 2024. Compares conventional, filter-based, data-driven and hybrid SOC methods. https://doi.org/10.1016/j.est.2024.113623

Electric vehicle battery capacity degradation and health estimation using machine-learning techniques: a review. Clean Energy, 2023. Focuses on SOH/RUL and ML approaches for EV battery ageing. https://doi.org/10.1093/ce/zkad047

A comprehensive review of machine learning-based state of health estimation for lithium-ion batteries: data, features, algorithms, and future challenges. Renewable and Sustainable Energy Reviews, 2025. Particularly useful for feature engineering, model selection and generalization issues. https://doi.org/10.1016/j.rser.2025.116125

State estimation for electric vehicles with deep learning- a survey. Journal of Power Sources, 2026. Broad survey covering SOC, SOH, SOE, SOP and temperature/state estimation. https://www.sciencedirect.com/science/article/pii/S0378779626000933

Machine learning is rewiring Li-ion battery RUL prediction: A systematic review and outlook. Journal of Energy Chemistry, 2026. Useful for RUL taxonomy, evaluation and current research gaps. https://doi.org/10.1016/j.jechem.2026.05.023

Physics-informed neural networks for battery modeling and state estimation: A methodological review. Renewable and Sustainable Energy Reviews, 2026. Excellent foundation for the project's physics-informed AI direction. https://doi.org/10.1016/j.rser.2026.117226

State-of-charge estimation of batteries using parameterized physics-informed neural networks. Journal of Energy Storage, 2026. Directly relevant to robust SOC estimation under diverse conditions and limited data. https://doi.org/10.1016/j.est.2026.122235

Physics-informed neural networks for electrode-level state estimation in lithium-ion batteries. Journal of Power Sources, 2021. Demonstrates physics-informed learning for internal battery state estimation. https://doi.org/10.1016/j.jpowsour.2021.230034

# 18. Data Source

NASA Li-ion Battery Aging Dataset: useful for reproducible ageing, SOH and RUL experiments. The dataset contains charge/discharge/EIS measurements under multiple operating profiles and temperatures.

https://data.nasa.gov/dataset/li-ion-battery-aging-datasets